HDMI (HDBaseT) Matrix Switcher Series

ITEM NO.:  HE04M

With the latest and more affordable HDBaseT technology extend HDMI sources up to 100M at 1080p or 1920x1200. HE04M HDMI 4 by 4 Matrix Switcher allows for effortless control of up to 4 devices through IR remote control, RS-232 and the front panel.

HE04M 4 x 4 HDMI (HDBaseT) Matrix Switcher

Feature:

- 4 x HDMI inputs , 4 x HDMI Outputs Matrix switcher

- Dual Output: local HDMI port or remote RJ45 termination

- Work with HE02ER, HE02R receiver at each monitor up to 100 meters over CAT5e/6.

- Work with HE02EIR receiver at each monitor up to 70 meters over CAT6a cable.

- Three control option: Front panel button, IR remote control, RS232 through remote PC.

- Support local/remote IR control to select input/output.

- Built in RS232 console port to select input/output.

- Uncompressed high definition video  1080p@60Hz 12 bit and 3D capable

- HDMI 1.4 video features supported.

- Built in IR extender (work with Receiver: HE02EIR)

- Built in RS232 extender ( work with Receiver: HE02R)

- Input/Output status could be displayed at panel LED.

- Optional Model:

HE02ER   100m HDMI Receiver

HE02R     100m HDMI & RS232 Receiver

HE02EIR   70m HDMI with IR Receiver

Panel View:

LED Indication:

Power		  　　ON/OFF

OUTPUT / INPUT:　 1，2，3，4

1 2,3,4 LEDS Flash: Select INPUT Channel

LED ON          : Input Selected

HDMI/RJ45 Output Selection:

Slide switch to right   HDMI OUT

Slide switch to left    Auto Detection HDMI or RJ45 output, HDMI priority when HDMI & RJ45 connection at the same time.

Input/Output Channel Switch:

- Select the output channel:  press top button (1，2，3，4), the selected column will LEDS flash

- Select the input channel:  press left button ( 1，2，3，4, OFF) to complete the channel selection.

Example 1 :  Input 1 switch to output 2, to press top button 2, then press left button 1.

Example 2 :  Turn off output 3 , to press top button 3 , then press left button OFF.

Local ( HE04M ) IR Control Function :

IR01 IR Remote Control Setting：

The ID number must be setup before using IR remote control or change new batteries.

Setup IR remote ID number: Press and hold “Power Symbol” button and press number 4.

Using IR01 remote controller to select input/output channel.

Input/Output Channel Switch:

- Select the output channel:  press IR button (1，2，3，4) the selected column will LEDS flash

- Select the input channel:  press IR button (7，8，9，＊, 0) to complete the channel selection.

1  Select Output 1

2  Select Output 2

3  Select Output 3

4  Select Output 4

7  Select Input 1

8  Select Input 2

9  Select Input 3

＊ Select Input 4

0  OFF

Remote IR Extender & HE04M Control Function :

IR01 IR Remote Control Setting：

The ID number must be setup before using IR remote control or change new batteries.

Setup IR remote ID number: Press and hold “Power Symbol”button and press number 5.

Work with HDMI source/monitor:

HE02EIR IR setting at TX, from local HE04M to control remote HDMI equipments.

HE02EIR IR setting at RX, from remote HDMI equipment to control local HE04M

Input/Output Channel Switch:

Select the input channel:  press IR button (1，2，3，4, 0) the selected column will LEDS flash

1  Select input 1

2  Select input 2

3  Select input 3

4  Select input 4

0  OFF

PC RS232 Port Remote Control (HE04M Console):

The communication format is 9600, 8, N, 1 (9600 bps, 1 start bit, 8 data bits, 1 stop bit).

Command List：

Use command as below：

Command + Space + Parameter + Enter

IN  OUT

1.  TX1  01   output 1 switch to input 1

2.  TX1  02   output 1 switch to input 2

3.  TX1  03   output 1 switch to input 3

4.  TX1  04   output 1 switch to input 4

5.  TX1  00   turn off output 1

6.  TX2  01   output 2  switch to input 1

7.  TX2  02   output 2  switch to input 2

8.  TX2  03   output 2  switch to input 3

9.  TX2  04   output 2  switch to input 4

10. TX2  00   turn off output 2

11. TX3  01   output 3 switch to input 1

12. TX3  02   output 3 switch to input 2

13. TX3  03   output 3 switch to input 3

14. TX3  04   output 3 switch to input 4

15. TX3  00   turn off output 3

16. TX4  01   output 4 switch to input 1

17. TX4  02   output 4 switch to input 2

18. TX4  03   output 4 switch to input 3

19. TX4  04   output 4 switch to input 4

20. TX4  00   turn off output 4

Installation View:

HE04MEK Package Include:

HE04M Matrix x 1 pcs

HE02ER HDMI Receiver x 4 pcs

IR01 IR Remote Control x 1 pcs

Power Adapter x 1 pcs

HE04MEIK Package Include:

HE04M Matrix x 1 pcs

HE02EIR HDMI Receiver x 4 pcs

IR01 IR Remote Control x 1 pcs

IR Emitter Cable x 4 pcs

IR Receiver Cable x 4 pcs

Power Adapter x 1 pcs

Specification:

| Item No. | HE04M |
| --- | --- |
| Max. Resolution | 1080p @60Hz 12bit |
| Video Input | HDMI Type A x 4 |
| Video Output | HDMI Type A x 4，RJ45 x4 |
| RJ45 Output Distance | 100 meters |
| IR Input/Output | Phone Jack x 2 |
| RS232 Input/Output | DB9 x 5 |
| Power Adapter | DC 12V 4Amp |
| Power Consumption | 2Amp (Max) |
| Temperature | Operation: 0 to 55℃, Storage: -20 to 85℃, Humidity: up to 95% |
| Dimension mm | 483 × 220 × 88.4 |
| Weight g | 3180 |

| Item No. | HE02ER (Optional CAT5E Receiver) | HE02EIR (Optional CAT5E Receiver) |
| --- | --- | --- |
| Resolution | Up to 1080p / 60Hz / 48 bit |  |
| Distance | 100M (Max) | 60M by CAT5e/6, 70M by CAT6a |
| Video Output | HDMI Type A x 1 |  |
| IR Carrier Frequency | X | 30-60KHz |
| IR Angle | X | ±55 |
| IR Distance | X | 5m |
| Power Supply | Regulated DC5V 2A |  |
| Power Consumption | 1200mA (Max) | 820mA (Max) |
| Temperature | Operation: 0 to 55℃, Storage: -20 TO 85℃, Humidity: up to 95% |  |
| Dimensions mm | 49 x 92.3 x 25 |  |
| Weight g | 135 | 135 |

Rev. A