IP CABLING TRANSMISSION – POE Switch

ITEM NO.:  IP06H 5 Port High Power POE Switch

IP06H PoE switch is specifically designed to meet IEEE 802.3at standards with support up to Gigabit speed and powering network devices. With PoE installation of network devices such as IP surveillance cameras, wireless access points, IP phones, and other PoE enabled devices in hard-to-reach, outdoor, and remote areas is simplified.

Features:

- 4-Port 10/100/1000Mbps with embedded PoE Injector.

- 1-Port 10/100/1000Mbps for connecting to LAN.

- Auto-sensing for detecting PoE or non-PoE devices.

- Compatible IEEE 802.3at PoE standard.

- 2 units install at TPN002 rack mounting become 8 ports Switch.

- Internal power supply.

- Option Model: TRN002 1U 19” Rack Mounting Panel

Installation View:

- Option Model: TRN002 1U 19” Rack Mounting Panel

Panel View:

IP06H

LED Indication:

| LED (Green) | ON | Power on |
| --- | --- | --- |
|  | OFF | Power off |
| Link/ACT (Yellow) | ON | 10/100/1000 Mbps link |
|  | OFF | 10/100/1000 Mbps unlink |
|  | FLASH | Activity |
| PoE (Green) | ON | PSE sending power |
|  | OFF | PSE non power |

Specification:

| ITEM NO. | IP06H |
| --- | --- |
| Input/Output Connector | RJ-45 x 5 |
| Data Link | Pins 1, 2 / 3, 6 / 4, 5 / 7, 8 |
| Power LinK | Pins 1, 2 / 3, 6 (End-Span) |
| LED Indicator | Power(Green), 10/100/1000/Act(Yellow), PoE(Green) |
| PoE Port Power | IEEE 802.3at Standard (1-Port 30W/600mA) |
| Power Supply | DC 54V 120W 2.22A |
| Power Consumption | Non-load 2.6W (48mA)，4port 4.1W (76mA)@10/100Mbps，4port 6.3W (116mA)@1000Mbps |
| Temperature | Operation：0 to 55°C, Storage：-20 to 85°C, Humidity：Up to 95% |
| Dimensions | 187 x 136.6 x 42 (mm) |
| Weight | 860 (g) |

Rev.A

FORESIGHT CCTV INC.

5863 Leslie Street, Unit#713, North York, Ontario, Canada, M2H 1J8

http://www.foresight-cctv.com | 1-888-GO2-CCTV