TWISTED PAIR TRANSMISSION SYSTEM – Passive Series

ITEM NO.: TTP111HD  HD Video Transceiver (BALUN)

Features:

 To send HD-CVI, HD-TVI, AHD signal 720p at 300 meters, 1080p at 200M .over UTP cable.

- BNC male to screw terminal for easy connection.

 Compact and mini size, could mount multiple units behind multiplexer, Quad, DVR..etc

  Passive, No power required.

  Eliminate Coaxial Cable.

  To work with Active transmitter TTA111HDT, Active receiver TTA111HDR for extra long distance transmission.

Panel View:

## Installation View

### Specification:

| ITEM NO. | TTP111HD |
| --- | --- |
| Video Input (BNC Connector) | 1 |
| Video Input | 1V p-p, 75 Ohms |
| CABLE | Twisted Pair CAT 5 |
| Bandwidth | 50Hz ~ 47MHz ( -3dB ) |
| Transmission Distance | Color: 300 meters |
| RJ-45 Output | Terminal |
| Power | X |
| Temperature | Operation: 0 to 55℃, Storage: -20 TO 85℃, Humidity: up to 95% |
| DIMENSIONS W x H x D mm | 14.5 × 49.5 × 15.5 |
| Weight g | 18 |