Online Coaxial / Twisted Pair Surge Protection Device

ITEM NO:  SP001, SP001VPD, SP004, SP016C,

Application for protects equipment from high voltage surges such as lightning strikes.  UL grounding requirements for coaxial cable offer protection from hazardous voltages that may occur on the shielding.  Because of the physical length of ground wires, UL grounding systems may appear as an open circuit to lightning caused, high frequency surges and provide no protection from these surges getting into a home and destroying CATV converters, TV sets, VCR, Camera etc.

FCC

Features:

- Performs a different function than grounding requirements.

- Protects equipment from high voltage surges from nearby lightning strikes.

- Two stage protection: 130V, AC/DC 12V

- Wide range of applications: CCTV system, Antenna of Television, satellite receivers, FM receivers, video camera, monitors, multiplexers & Quad processor, VCR, etc.

SP001   Coaxial Surge Protection Device BNC Connector

BNC male to BNC female. Hi Frequency for both CCTV and CATV use.

SP001VPD   Video, Power, Data Surge Protection

5 pin terminal + BNC female to 4 pin terminal + BNC female.

Two stage protection, AC/DC 130V, AC/DC 12V

Max power input AC 40V(MAX), DC 56V(MAX).

SP004   UTP Data Surge protection Device Terminal Connector

Terminal connector to terminal connector.

Perfect for video or data signal protection

SP016C   Video Surge Protection For DVR in 1U Rack Mounting Panel

16 BNC female to 16 BNC female,

Provide protection for DVR, multiplexers or any equipment with multiple video inputs.

Installation View:

SP001

SP001VPD

SP004

SP016C

SP016C

Application

Note:

For ensuring the unit can protect equipments, the cable must be connected to ground.

- CATV System

- Antenna of Television

- Video Cassette Recorders

- FM receivers

- Video Cameras and monitors

- Multiplexers and Quad Processor

Specification:

| ITEM NO. | SP001 | SP001VPD | SP004 | SP016C |
| --- | --- | --- | --- | --- |
| Surge Protection Attack Time | 1 ns |  |  |  |
| Maximum Surge Current | 4 KV |  |  |  |
| Surge Life | 300 surges of 100Amps |  |  |  |
| Normal Resistance Across Coax | 10,000 MΩ |  |  |  |
| Surge Resistance | Approx. 0Ω |  |  |  |
| BAND PASS | 0~1 GHZ |  |  |  |
| Power Passing Capability | 130V AC / DC, 65 Vrms at10 Amps |  |  |  |
| Insertion Loss | Less than 0.2 dB |  |  |  |
| Return Loss (75 ohms) | 16dB |  |  |  |
| Power IN Put – for SP001VPD | AC 40V(MAX), DC56V(MAX) |  |  |  |
| Connectors | BNC | Terminal | Terminal | BNC |
| Dimensions W x H x D mm | 78x25x25 | 87x67x27 | 65x25x25 | 482x170x 44mm standard 1U Rack Panel |
| WEIGHT g | 80 | 136 | 65 | 1200 |