Online Coaxial Surge Protection Device

ITEM NO:  SP001, SP001P

Application for protects equipment from high voltage surges such as lightning strikes.  UL grounding requirements for coaxial cable offer protection from hazardous voltages that may occur on the shielding.  Because of the physical length of ground wires, UL grounding systems may appear as an open circuit to lightning caused, high frequency surges and provide no protection from these surges getting into a home and destroying CATV converters, TV sets, VCR, Camera etc.

FCC

Features:

- Performs a different function than grounding requirements.

- Protects equipment from high voltage surges from nearby lightning strikes.

- Two stage protection: 130V, AC/DC 12V

- Wide range of applications: CCTV system, Antenna of Television, satellite receivers, FM receivers, video camera, monitors, multiplexers & Quad processor, VCR, etc.

SP001   Coaxial Surge Protection Device BNC Connector

BNC male to BNC female. Hi Frequency for both CCTV and CATV use.

SP001P   Power Surge protection Device

2 pin terminal for power input to 2 pin terminal for power output.

Two stage protection, AC/DC 130V

Max power input AC 40V(MAX), DC 56V(MAX)

Installation View:

SP001

SP001P

Application

Note:

For ensuring the unit can protect equipments, the cable must be connected to ground.

- CATV System

- Antenna of Television

- Video Cassette Recorders

- FM receivers

- Video Cameras and monitors

- Multiplexers and Quad Processor

Specification:

| ITEM NO. | SP001 |
| --- | --- |
| Surge Protection Attack Time | 1 ns |
| Maximum Surge Current | 4 KV |
| Surge Life | 300 surges of 100Amps |
| Normal Resistance Across Coax | 10,000 MΩ |
| Surge Resistance | Approx. 0Ω |
| BAND PASS | 0~1 GHZ |
| Power Passing Capability | 130V AC / DC, 65 Vrms at10 Amps |
| Insertion Loss | Less than 0.2 dB |
| Return Loss (75 ohms) | 16dB |
| Connectors | BNC |
| Dimensions W x H x D mm | 78x25x25 |
| WEIGHT g | 80 |

| ITEM NO. | SP001P |
| --- | --- |
| Surge Protection Attack Time | 1 ns |
| Maximum Surge Voltage | 4 KV |
| Surge Life | 300 surges of 100Amps |
| Normal Resistance Across Coax | 10,000 MΩ |
| Surge Resistance | Approx. 0Ω |
| BAND PASS | 0~1 GHZ |
| Power Passing Capability | 130V DC / AC, 65 Vrms at10 Amps |
| Insertion Loss | Less than 0.2 dB |
| Return Loss (75 ohms) | 16dB |
| Power IN Put | AC 40V(MAX), DC56V(MAX) |
| Connectors | Terminal |
| Dimensions W x H x D mm | 65x25x25 |
| WEIGHT g | 65 |